# ABB Ports Supplier Component Requirements

Purpose. This document states the minimum requirements that a supplier component shall meet before it can be offered for ABB Ports equipment. The supplier shall confirm compliance for the intended installation zone and identify any deviations before quotation or delivery.

# 1. Scope

Applies to electrical, electronic and mechanical components, cabinets, sensor systems, video systems, network components, power supplies, cables, PCBAs and higher-level assemblies used in ABB Marine and Ports applications. Site-specific ABB drawings, engineering technical specifications or purchase-order notes can add requirements and take precedence for that delivery.

# 2. Installation zones

| Zone | Environment | Typical locations |
| --- | --- | --- |
| A | Indoor, climate controlled, protected from weather, lowest vibration/shock. | Control room, office spaces, HVAC areas |
| B | Outdoor weather exposure with moderate crane vibration. | Boom landside, gantry legs, machinery house |
| C | Outdoor weather exposure with high sustained vibration and moderate shock. | Trolley frame, festoon systems, cable reels, waterside boom near hinge |
| D | Outdoor weather exposure with highest mechanical shock from container handling. | Spreader frame, twist locks, spreader sensors, headblock |

Zone C and Zone D are not interchangeable. Zone C is mainly a vibration environment; Zone D is mainly a shock environment. A component is acceptable only for the zone(s) it has been qualified against.

# 3. Minimum requirements by zone

| Requirement | Zone A | Zone B | Zone C | Zone D |
| --- | --- | --- | --- | --- |
| Operating temperature | +5 to +40 °C | -20 to +55 °C | -20 to +55 °C | -20 to +55 °C |
| Storage temperature | +5 to +50 °C | -30 to +70 °C | -30 to +70 °C | -30 to +70 °C |
| Ingress protection | IP2X | IP66 | IP66 | IP66 |
| Humidity | 40 °C, 93 %RH, 4 d | 40 °C, 93 %RH, 10 d | 40 °C, 93 %RH, 10 d | 40 °C, 93 %RH, 10 d |
| Salt / corrosion | N/A | 1000 h neutral salt exposure, high-durability coating | 1000 h neutral salt exposure, high-durability coating | 1000 h neutral salt exposure, high-durability coating |
| Sinusoidal vibration | 0.5 g, 10-150 Hz | 1.5 g, 10-500 Hz | 3.0 g, 5-500 Hz | 3.0 g, 10-500 Hz |
| Random vibration | 0.5 g RMS | 0.8 g RMS | 1.5 g RMS | 2.0 g RMS |
| Mechanical shock | 3 g, 11 ms | 5 g, 11 ms | 8 g, 11 ms | 50 g, 11 ms |
| Repetitive shock / bump | N/A | 5 g, 1000/axis | 8 g, 2000/axis | 30 g, 4000/axis |
| UV resistance | N/A | 2000 h | 2000 h | 2000 h |

# 4. Common electrical and EMC requirements

| Item | Minimum requirement |
| --- | --- |
| Supply voltage | 110-230 VAC ±15 %, 1-phase, 50/60 Hz, unless otherwise specified |
| Control voltage | 12-48 VDC ±20 %, unless otherwise specified |
| Generic EMC immunity | Suitable for industrial port environment |
| Generic EMC emission | Suitable for industrial port environment |
| ESD immunity | ±8 kV contact, ±15 kV air |
| Surge immunity | 4 kV line-earth, 2 kV line-line |

# 5. Materials, mounting and documentation

Outdoor components shall use marine-grade corrosion-resistant materials, high-durability coated carbon steel, marine aluminium, UV-stabilized polymers and ozone-resistant elastomers as applicable.

Zone C components shall use locking connectors and suitable strain relief; vibration damping shall be considered for components mounted near the boom hinge or trolley.

Zone D components shall use wedge-locking fasteners or equivalent, screw-lock/bayonet-lock connectors and strain relief sized for high shock exposure.

Supplier shall maintain an appropriate quality management system, full traceability and documented inspection/test plans.

Delivery documentation shall include Certificate of Conformity stating zone rating, relevant test reports, material certificates where applicable, coating reports, installation instructions and maintenance instructions.

# 6. Supplier confirmation

By quoting or delivering against this requirement document, the supplier confirms that the component complies with the specified installation zone(s). Deviations shall be documented with technical justification and proposed mitigation before ABB approval. Minimum warranty shall be 24 months from commissioning or 30 months from delivery, whichever occurs first.